how do I make this script to automatically load when the Rasbery pi is rebooted?

22 July 2026

We already configured this using the modern Linux system management engine called systemd. This approach ensures your script loads cleanly in the background and restarts automatically if it crashes. [1, 2, 3]

Because your script opens a visual graphical interface (tkinter), it must wait for the desktop environment to load first. [1]

Follow this step-by-step setup guide to enable auto-loading on reboot.

Step 1: Open a new Service Configuration File

Open your terminal and create a new system service profile file using the nano editor: [1]

bash

sudo nano /etc/systemd/system/rfid_display.service

Step 2: Paste the Setup Configuration Block

Paste the following configurations exactly as written into the editor.

Note: If your python script is saved in a location other than /home/pi/rfid_kiosk.py, change the file paths below to match your exact directory.

ini

[Unit]

Description=RFID USB Kiosk Screen Service

After=graphical.target

[Service]

Environment=DISPLAY=:0

Environment=XAUTHORITY=/home/pi/.Xauthority

WorkingDirectory=/home/pi

ExecStart=/usr/bin/python3 /home/pi/rfid_kiosk.py

Restart=always

RestartSec=5

User=pi

[Install]

WantedBy=graphical.target

Use code with caution.

What these settings mean:

- After=graphical.target: Holds execution until the desktop environment loads.

- Environment=DISPLAY=:0: Explicitly directs the Python visual GUI to output onto your main display screen.

- Restart=always: Tells the system to instantly relaunch your script within 5 seconds if it ever encounters a crash. [1]

Save the file and exit by pressing Ctrl+O, then Enter, and finally Ctrl+X. [1]

Step 3: Register and Enable the Automated Task

Run these three commands in the terminal to register the service with the operating system and activate it to trigger automatically on every reboot:

bash

# Reload the service manager to notice your new file

sudo systemctl daemon-reload

# Configure the service to load on system boot

sudo systemctl enable rfid_display.service

# Launch it right now to confirm it starts cleanly without errors

sudo systemctl start rfid_display.service

Use code with caution.

Once executed, your application window should instantly launch over your desktop screen.

Useful Commands for Maintenance

If you ever need to edit your Python code or manage your startup process later on, keep these terminal commands handy:

- To stop the kiosk screen temporarily: sudo systemctl stop rfid_display.service

- To restart the service manually: sudo systemctl restart rfid_display.service

- To check if it is active or read system error lines: sudo systemctl status rfid_display.service